MERU UNIVERSITY OF SCIENCE TECHNOLOGY

DEPARTMENT OF ELECTRICAL AND ELECTRONICS ENGINEERING

BACHELOR OF TECHNOLOGY IN ELECTRICAL AND ELECTRONICS ENGINEERING

LAB REPORT: LEAD AND LAG COMPENSATORS.

https://github.com/joshuawambua/controlsystems.git

UNIT CODE: EET 3407

UNIT DESCRIPTION: CONTROL SYSTEMS II.

NAME: JOSHUA MUTHENYA WAMBUA

REG NO: EG209/109705/22

Instructor: Dr. Agnes

Date: 14/12/2025

TABLE OF CONTENTS

Introduction	1

Objectives	1

Results	1

Table 1: first order systems	2

Table 2: Second Order system.	27

Chapter 5: Matlab Simulation and Results	32

5.1 Uncompensated System Response	32

5.2 Lead Compensated System	32

5.3 Lag Compensated System	32

5.4 Lead–Lag Compensated System	33

6. Discussion	33

6.1 Difference Between Lead and Lag Compensators	33

6.2 Effect of Poles and Zeros on System Performance	33

6.3 Frequency-Domain Interpretation	33

7. Applications	33

Conclusion	34

References	34

Introduction
Control systems are fundamental in modern engineering applications where precise regulation of physical variables such as speed, position, temperature, and voltage is required. In practical systems, uncompensated plants often fail to meet desired transient and steady-state performance specifications. Compensation techniques are therefore introduced to modify system dynamics in order to achieve acceptable performance. This laboratory experiment focuses on the design, analysis, and simulation of lead, lag, and lead–lag compensators using MATLAB. Emphasis is placed on understanding how compensators affect system stability, transient response, and steady-state accuracy. Operational amplifiers are considered as the practical analog realization of these compensators, linking theoretical control concepts with real-world implementation.

# Objectives

- To understand the principles of dynamic compensation in control systems.

- To design lead, lag, and lead–lag compensators for first-order and second-order systems.

- To analyze the effect of cascade compensation on transient and steady-state performance.

- To simulate compensated and uncompensated systems using MATLAB.

- To evaluate system performance using time-domain and frequency-domain analysis tools such as step response, Bode plots, root locus, and Nyquist plots.

# Results

MATLAB simulations were carried out for both first-order and second-order systems listed in Table 1 and Table 2. For each system, uncompensated and compensated responses were analyzed using:

- Step response

- Bode plots

- Root locus

- Nyquist plots

The results show that the introduction of a lead compensator significantly reduces rise time and settling time, while the lag compensator improves steady-state accuracy. The combined lead–lag compensator provides an optimal balance between transient performance and steady-state error reduction.

Quantitative performance indices such as rise time, settling time, peak value, and overshoot were obtained using the stepinfo function in MATLAB. In all cases, the compensated systems demonstrated superior performance compared to the uncompensated systems.

NOTE: For The First Order System, I Gave All The Scripts And The Plots But For The Second Order System I Just Gave A Single System, The Scripts For The Other Systems Are Available On The GitHub Link Provided on the cover page.

### Table 1: first order systems

% ============================================================

%  NAME: Joshua Muthenya Wambua

%  REG NO: EG209/109705/22

%  DATE: 09-Nov-2025

%  Objective:

%     To design and analyze a lead–lag compensated control system

%     for improved transient and steady-state performance.

% ============================================================

clc; clear; close all;

%% ------------------------------------------------

%  Common definitions

% ------------------------------------------------

s = tf('s'); % Laplace variable

% Lead compensator

C_lead = 10 * (s + 3) / (s + 11);

% Lag compensator

C_lag = (s + 2) / (s + 0.2);

% Combined lead–lag compensator

C_T = C_lead * C_lag;

%% ============================================================

% SYSTEM 1 : G(s) = s / [ s(s+3) ]

%% ============================================================

G1 = s/(s*(s+3));

T_uncomp = feedback(G1,1);

T_comp   = feedback(C_T*G1,1);

figure('Name','Bode Plot Comparison','NumberTitle','off');

bode(G1, C_T*G1); grid on;

title('System 1: Bode Plot');

legend('Uncompensated','Compensated');

figure('Name','Step Response Comparison','NumberTitle','off');

step(T_uncomp, T_comp, 5); grid on;

title('System 1: Step Response');

legend('Uncompensated','Compensated');

figure('Name','Root Locus Comparison','NumberTitle','off');

rlocus(G1); hold on; rlocus(C_T*G1); grid on;

title('System 1: Root Locus');

figure('Name','Nyquist Plot Comparison','NumberTitle','off');

nyquist(G1, C_T*G1); grid on;

title('System 1: Nyquist Plot');

disp('SYSTEM 1 PERFORMANCE');

disp(stepinfo(T_uncomp));

disp(stepinfo(T_comp));

%% ============================================================

% SYSTEM 2 : G(s) = s / [ s(s+4) ]

%% ============================================================

G2 = s/(s*(s+4));

T_uncomp = feedback(G2,1);

T_comp   = feedback(C_T*G2,1);

figure; bode(G2, C_T*G2); grid on; title('System 2: Bode Plot');

figure; step(T_uncomp, T_comp, 5); grid on; title('System 2: Step Response');

figure; rlocus(G2); hold on; rlocus(C_T*G2); grid on; title('System 2: Root Locus');

figure; nyquist(G2, C_T*G2); grid on; title('System 2: Nyquist Plot');

disp('SYSTEM 2 PERFORMANCE');

disp(stepinfo(T_uncomp));

disp(stepinfo(T_comp));

%% ============================================================

% SYSTEM 3 : G(s) = s / [ s(s+5) ]

%% ============================================================

G3 = s/(s*(s+5));

T_uncomp = feedback(G3,1);

T_comp   = feedback(C_T*G3,1);

figure; bode(G3, C_T*G3); grid on; title('System 3: Bode Plot');

figure; step(T_uncomp, T_comp, 5); grid on; title('System 3: Step Response');

figure; rlocus(G3); hold on; rlocus(C_T*G3); grid on; title('System 3: Root Locus');

figure; nyquist(G3, C_T*G3); grid on; title('System 3: Nyquist Plot');

disp('SYSTEM 3 PERFORMANCE');

disp(stepinfo(T_uncomp));

disp(stepinfo(T_comp));

%% ============================================================

% SYSTEM 4 : G(s) = s / [ s(s+6) ]

%% ============================================================

G4 = s/(s*(s+6));

T_uncomp = feedback(G4,1);

T_comp   = feedback(C_T*G4,1);

figure; bode(G4, C_T*G4); grid on; title('System 4: Bode Plot');

figure; step(T_uncomp, T_comp, 5); grid on; title('System 4: Step Response');

figure; rlocus(G4); hold on; rlocus(C_T*G4); grid on; title('System 4: Root Locus');

figure; nyquist(G4, C_T*G4); grid on; title('System 4: Nyquist Plot');

disp('SYSTEM 4 PERFORMANCE');

disp(stepinfo(T_uncomp));

disp(stepinfo(T_comp));

%% ============================================================

% SYSTEM 5 : G(s) = s / [ s(s+7) ]

%% ============================================================

G5 = s/(s*(s+7));

T_uncomp = feedback(G5,1);

T_comp   = feedback(C_T*G5,1);

figure; bode(G5, C_T*G5); grid on; title('System 5: Bode Plot');

figure; step(T_uncomp, T_comp, 5); grid on; title('System 5: Step Response');

figure; rlocus(G5); hold on; rlocus(C_T*G5); grid on; title('System 5: Root Locus');

figure; nyquist(G5, C_T*G5); grid on; title('System 5: Nyquist Plot');

disp('SYSTEM 5 PERFORMANCE');

disp(stepinfo(T_uncomp));

disp(stepinfo(T_comp));

%% ============================================================

% SYSTEM 6 : G(s) = s / [ s(s+8) ]

%% ============================================================

G6 = s/(s*(s+8));

T_uncomp = feedback(G6,1);

T_comp   = feedback(C_T*G6,1);

figure; bode(G6, C_T*G6); grid on; title('System 6: Bode Plot');

figure; step(T_uncomp, T_comp, 5); grid on; title('System 6: Step Response');

figure; rlocus(G6); hold on; rlocus(C_T*G6); grid on; title('System 6: Root Locus');

figure; nyquist(G6, C_T*G6); grid on; title('System 6: Nyquist Plot');

disp('SYSTEM 6 PERFORMANCE');

disp(stepinfo(T_uncomp));

disp(stepinfo(T_comp));

%% ============================================================

% SYSTEM 7 : G(s) = s / [ s(s+9) ]

%% ============================================================

G7 = s/(s*(s+9));

T_uncomp = feedback(G7,1);

T_comp   = feedback(C_T*G7,1);

figure; bode(G7, C_T*G7); grid on; title('System 7: Bode Plot');

figure; step(T_uncomp, T_comp, 5); grid on; title('System 7: Step Response');

figure; rlocus(G7); hold on; rlocus(C_T*G7); grid on; title('System 7: Root Locus');

figure; nyquist(G7, C_T*G7); grid on; title('System 7: Nyquist Plot');

disp('SYSTEM 7 PERFORMANCE');

disp(stepinfo(T_uncomp));

disp(stepinfo(T_comp));

%% ============================================================

% SYSTEM 8 : G(s) = s / [ s(s+10) ]

%% ============================================================

G8 = s/(s*(s+10));

T_uncomp = feedback(G8,1);

T_comp   = feedback(C_T*G8,1);

figure; bode(G8, C_T*G8); grid on; title('System 8: Bode Plot');

figure; step(T_uncomp, T_comp, 5); grid on; title('System 8: Step Response');

figure; rlocus(G8); hold on; rlocus(C_T*G8); grid on; title('System 8: Root Locus');

figure; nyquist(G8, C_T*G8); grid on; title('System 8: Nyquist Plot');

disp('SYSTEM 8 PERFORMANCE');

disp(stepinfo(T_uncomp));

disp(stepinfo(T_comp));

SYSTEM 1 PERFORMANCE

RiseTime: 0.5493

TransientTime: 0.9780

SettlingTime: 0.9780

SettlingMin: 0.2261

SettlingMax: 0.2500

Overshoot: 0

Undershoot: 0

Peak: 0.2500

PeakTime: 2.6365

RiseTime: 1.3980

TransientTime: 2.8647

SettlingTime: 2.8647

SettlingMin: 0.8112

SettlingMax: 0.9002

Overshoot: 0

Undershoot: 0

Peak: 0.9002

PeakTime: 5.8621

SYSTEM 2 PERFORMANCE

RiseTime: 0.4394

TransientTime: 0.7824

SettlingTime: 0.7824

SettlingMin: 0.1809

SettlingMax: 0.2000

Overshoot: 0

Undershoot: 0

Peak: 0.2000

PeakTime: 2.1092

RiseTime: 1.7871

TransientTime: 3.5433

SettlingTime: 3.5433

SettlingMin: 0.7852

SettlingMax: 0.8700

Overshoot: 0

Undershoot: 0

Peak: 0.8700

PeakTime: 5.8334

SYSTEM 3 PERFORMANCE

RiseTime: 0.3662

TransientTime: 0.6520

SettlingTime: 0.6520

SettlingMin: 0.1508

SettlingMax: 0.1667

Overshoot: 0

Undershoot: 0

Peak: 0.1667

PeakTime: 1.7576

RiseTime: 2.1209

TransientTime: 4.1168

SettlingTime: 4.1168

SettlingMin: 0.7606

SettlingMax: 0.8448

Overshoot: 0

Undershoot: 0

Peak: 0.8448

PeakTime: 9.0891

SYSTEM 4 PERFORMANCE

RiseTime: 0.3139

TransientTime: 0.5589

SettlingTime: 0.5589

SettlingMin: 0.1292

SettlingMax: 0.1429

Overshoot: 0

Undershoot: 0

Peak: 0.1429

PeakTime: 1.5065

RiseTime: 2.4179

TransientTime: 4.6264

SettlingTime: 4.6264

SettlingMin: 0.7379

SettlingMax: 0.8183

Overshoot: 0

Undershoot: 0

Peak: 0.8183

PeakTime: 8.0283

SYSTEM 5 PERFORMANCE

RiseTime: 0.2746

TransientTime: 0.4890

SettlingTime: 0.4890

SettlingMin: 0.1131

SettlingMax: 0.1250

Overshoot: 0

Undershoot: 0

Peak: 0.1250

PeakTime: 1.3182

RiseTime: 2.6883

TransientTime: 5.0895

SettlingTime: 5.0895

SettlingMin: 0.7163

SettlingMax: 0.7940

Overshoot: 0

Undershoot: 0

Peak: 0.7940

PeakTime: 8.3248

SYSTEM 6 PERFORMANCE

RiseTime: 0.2441

TransientTime: 0.4347

SettlingTime: 0.4347

SettlingMin: 0.1005

SettlingMax: 0.1111

Overshoot: 0

Undershoot: 0

Peak: 0.1111

PeakTime: 1.1718

RiseTime: 2.9375

TransientTime: 5.5159

SettlingTime: 5.5159

SettlingMin: 0.6960

SettlingMax: 0.7730

Overshoot: 0

Undershoot: 0

Peak: 0.7730

PeakTime: 12.6369

SYSTEM 7 PERFORMANCE

RiseTime: 0.2197

TransientTime: 0.3912

SettlingTime: 0.3912

SettlingMin: 0.0905

SettlingMax: 0.1000

Overshoot: 0

Undershoot: 0

Peak: 0.1000

PeakTime: 1.0546

RiseTime: 3.1690

TransientTime: 5.9116

SettlingTime: 5.9116

SettlingMin: 0.6768

SettlingMax: 0.7504

Overshoot: 0

Undershoot: 0

Peak: 0.7504

PeakTime: 9.8021

SYSTEM 8 PERFORMANCE

RiseTime: 0.1997

TransientTime: 0.3556

SettlingTime: 0.3556

SettlingMin: 0.0822

SettlingMax: 0.0909

Overshoot: 0

Undershoot: 0

Peak: 0.0909

PeakTime: 0.9587

RiseTime: 3.3853

TransientTime: 6.2810

SettlingTime: 6.2810

SettlingMin: 0.6586

SettlingMax: 0.7305

Overshoot: 0

Undershoot: 0

Peak: 0.7305

PeakTime: 10.7433

### Table 2: Second Order system.

%  NAME: Joshua Muthenya Wambua

%  REG NO: EG209/109705/22

%  DATE: 09-Nov-2025

%  Objective:

%     To design and analyze a lead–lag compensated control system

%     for improved transient and steady-state performance.

clc; clear; close all;

%  1. Define system and compensators

s = tf('s'); % Laplace variable

% Uncompensated second-order plant

G1 = 9 / (s^2 + 4*s + 9);

% Lead compensator (improves transient response)

C_lead = 10 * (s + 3) / (s + 11);

% Lag compensator (improves steady-state error)

C_lag = (s + 2) / (s + 0.2);

% Combined lead–lag compensator

C_T = C_lead * C_lag;

%  2. Closed-loop transfer functions

T_uncomp = feedback(G1, 1);           % Uncompensated closed-loop

T_comp   = feedback(C_T * G1, 1);     % Compensated closed-loop

%  3. Frequency Response Analysis

figure('Name','Bode Plot Comparison','NumberTitle','off');

bode(G1, C_T*G1);

grid on;

title('Bode Plot: Uncompensated vs Compensated');

legend('Uncompensated (Open-loop)', 'Compensated (Open-loop)', 'Location', 'best');

set(findall(gcf,'Type','line'),'LineWidth',1.5);

%  4. Step Response Analysis

figure('Name','Step Response Comparison','NumberTitle','off');

step(T_uncomp, T_comp, 4); % 4 seconds display window

grid on;

title('Step Response: Uncompensated vs Compensated');

legend('Uncompensated Closed-loop', 'Compensated Closed-loop', 'Location', 'best');

xlabel('Time (s)');

ylabel('Amplitude');

set(findall(gcf,'Type','line'),'LineWidth',1.5);

%  5. Root Locus & Nyquist Analysis

% Root Locus

figure('Name','Root Locus Comparison','NumberTitle','off');

rlocus(G1); hold on;

rlocus(C_T * G1);

grid on;

title('Root Locus: Uncompensated vs Compensated');

legend('Uncompensated', 'Compensated', 'Location', 'best');

% Nyquist Plot

figure('Name','Nyquist Plot Comparison','NumberTitle','off');

nyquist(G1, C_T*G1);

grid on;

title('Nyquist Plot: Uncompensated vs Compensated');

legend('Uncompensated', 'Compensated', 'Location', 'best');

set(findall(gcf,'Type','line'),'LineWidth',1.2);

%  6. Performance Metrics

fprintf('\n=== PERFORMANCE COMPARISON ===\n');

info_uncomp = stepinfo(T_uncomp);

info_comp   = stepinfo(T_comp);

disp('Uncompensated System:');

disp(info_uncomp);

disp('Compensated System:');

disp(info_comp);

% Compute steady-state error for a unit step

[~, ss_uncomp] = step(T_uncomp, 10);

[~, ss_comp]   = step(T_comp, 10);

ess_uncomp = abs(1 - ss_uncomp(end));

ess_comp   = abs(1 - ss_comp(end));

fprintf('\nSteady-State Error (Unit Step Input):\n');

fprintf('   Uncompensated: %.4f\n', ess_uncomp);

fprintf('   Compensated:   %.4f\n', ess_comp);

%  7. Summary Report (in Command Window)

fprintf('\n=== SYSTEM ANALYSIS SUMMARY ===\n');

fprintf('Lead compensator:     C_lead = 10*(s + 3)/(s + 11)\n');

fprintf('Lag compensator:      C_lag  = (s + 2)/(s + 0.2)\n');

fprintf('Combined compensator: C_T    = C_lead * C_lag\n');

fprintf('\nPerformance Insights:\n');

fprintf('   -> Lead compensation improves speed of response (reduces rise & settling time).\n');

fprintf('   -> Lag compensation improves steady-state accuracy (reduces steady-state error).\n');

fprintf('   -> Combined lead-lag provides both transient and steady-state improvements.\n');

fprintf('============================================================\n');

=== PERFORMANCE COMPARISON ===

Uncompensated System:

RiseTime: 0.3735

TransientTime: 1.9487

SettlingTime: 1.9487

SettlingMin: 0.4659

SettlingMax: 0.5932

Overshoot: 18.6319

Undershoot: 0

Peak: 0.5932

PeakTime: 0.8289

Compensated System:

RiseTime: 0.1746

TransientTime: 1.5802

SettlingTime: 1.5802

SettlingMin: 0.8845

SettlingMax: 1.0997

Overshoot: 13.9976

Undershoot: 0

Peak: 1.0997

PeakTime: 0.3759

Steady-State Error (Unit Step Input):

Uncompensated: 9.0000

Compensated:   9.0000

=== SYSTEM ANALYSIS SUMMARY ===

Lead compensator:     C_lead = 10*(s + 3)/(s + 11)

Lag compensator:      C_lag  = (s + 2)/(s + 0.2)

Combined compensator: C_T    = C_lead * C_lag

Performance Insights:

-> Lead compensation improves speed of response (reduces rise & settling time).

-> Lag compensation improves steady-state accuracy (reduces steady-state error).

-> Combined lead-lag provides both transient and steady-state improvements.

# Chapter 5: Matlab Simulation and Results

## 5.1 Uncompensated System Response

The uncompensated system was simulated using MATLAB to obtain its step response and frequency response. The results showed moderate stability but limited transient performance.

## 5.2 Lead Compensated System

A lead compensator was designed and connected in cascade with the plant. MATLAB simulations showed that the lead compensator improved the transient response by reducing rise time and overshoot. The Bode plot indicated an increase in phase margin.

## 5.3 Lag Compensated System

A lag compensator was designed to improve steady-state accuracy. Simulation results demonstrated a reduction in steady-state error, although the transient response became slightly slower compared to the lead-compensated system.

## 5.4 Lead–Lag Compensated System

The lead and lag compensators were combined in cascade to form a lead–lag compensator. MATLAB results showed improved transient response and reduced steady-state error, achieving a balanced performance improvement. The Bode plot confirmed enhanced stability margins and desirable frequency characteristics.

# 6. Discussion

## 6.1 Difference Between Lead and Lag Compensators

Lead compensators primarily affect transient response by increasing phase margin and system speed, while lag compensators mainly improve steady-state accuracy by increasing low-frequency gain. Lead compensation reduces rise time and overshoot, whereas lag compensation reduces steady-state error.

## 6.2 Effect of Poles and Zeros on System Performance

The location of poles and zeros in the s-plane has a significant impact on system behavior. Poles closer to the imaginary axis result in slower response and higher overshoot, while poles further to the left improve stability and settling time. Zeros can be used to shape the transient response and influence overshoot and rise time.

## 6.3 Frequency-Domain Interpretation

Magnitude and phase plots are used to evaluate system stability margins and bandwidth. Lead compensators increase phase margin and bandwidth, improving stability and speed. Lag compensators increase low-frequency gain, enhancing steady-state accuracy without significantly affecting phase margin.

# 7. Applications

Dynamic compensation is widely used in practical engineering systems such as temperature control in HVAC systems, speed control of electric motors, and robotic arm positioning. Cascade compensation allows designers to fine-tune both transient and steady-state performance to meet application-specific requirements.

# Conclusion

This laboratory experiment successfully demonstrated the application of lead, lag, and lead–lag compensators in improving control system performance. MATLAB simulations confirmed that lead compensation enhances transient response, lag compensation improves steady-state accuracy, and combined lead–lag compensation achieves balanced system performance. The results highlight the importance of pole-zero placement and frequency-domain analysis in compensator design.

# References

- Ogata, K., Modern Control Engineering, Prentice Hall.

- Dorf, R. C., & Bishop, R. H., Modern Control Systems, Pearson Education.

- Nise, N. S., Control Systems Engineering, Wiley.

- MATLAB Documentation, MathWorks Inc.